INTERNSHIP REPORT

AI BASED HEADWAY OPTIMIZATION FOR METRO LINE 4, Mumbai

A Report Submitted in Partial Fulfillment of the Requirements for The Degree Of

Bachelor Of Technology

By

NAME: Amrit Bhardwaj                                                              Roll no: 23EC008

to the

School of Technology

GATI SHAKTI VISHWAVIDYALAYA

Central University, Under Ministry of Railways

Vadodara, India-390004

Date: 18/06/2025

SUMMER INTERNSHIP

CERTIFICATE OF COMPLETION

This Is to Certify That the Work Contained in The Project Titled “Ai Based Headway Optimization for Line 4” Has Successfully Completed by the Student Under My Supervision During 5 Week Summer Internship. During The Internship Period the Students Found to Be Hard Working, Punctual and Discipline and He Met All the Criteria Satisfactorily.

STUDENT NAME                                   ROLL NUMBER                        SIGNATURE

Amrit Bhardwaj                                     23EC008

MENTOR DETAILS:

MR RASHMI RANJAN PATTNAIK

SIGNATURE

Acknowledgement

I take this opportunity to express my sincere thanks to my mentor Mr Rashmi Ranjan Patnaik for their guidance and support. I would also like to express my gratitude towards them for showing confidence in me. It was a privilege to have a great experience working under him in a cordial environment.

I am very much thankful to the MMRDA, for providing me the opportunity of pursuing summer internship in Mumbai metro line 4 General consultant I am very much thankful to MMRDA    for providing funds to do summer internship.

I am thankful to office staffs and other relevant persons who helped me to figure out and give a friendly environment.

Name

Amrit Bhardwaj

Abstract

This project focuses on headway optimization for Metro Line 4 using artificial intelligence. The core issue which is faced in the metro is cascading delays experienced by the metro trains, which is generally caused by increasing dwell time. Dwell time refers to the amount of time a train is at rest to allow a passenger to get on board and alight. Now, experiencing this dwell time in crowded cities like Mumbai during peak hours, the dwell time increases, and it leads to serial delays arising to the trains moving before it. This is because the current CBTC (Communication-Based Train Control) system, which is being employed in most metro networks—including Line 4—has a feature of using a fixed headway model. Headway is defined as the minimum time interval between two successive trains running on the same track, say upwards or downwards. It is calculated using the formula:

Headway = Run Time + Dwell Time + Crossover Time,
followed by division with the number of trains (rolling stock) available on the line.

The formula is not dependent on any real factor which is varying in operational measures and is revised every year based on the actual headway achieved. Fixed headway limits the responsiveness of the system to this load, especially during peak hours.

This leads to using some method or technology so that we can optimize the headway such that cascading delay could be reduced, and here comes artificial intelligence into the picture.

So, the metro needs a method where they could minimize the headway in such a manner that the safest headway is also not breached, and we could optimize the headway that is calculated from the formulas but also takes into account the new factors that cause headway issues.

To illustrate this, consider a train departing from Gaimukh at 8:00 AM and reaching Gowniwada at 8:05 AM, with a scheduled headway of 10 minutes. If the train is delayed by 2 minutes at Gowniwada and incurs a further delay of 2 minutes to the next train—originally expected to arrive at 8:15 AM—may now arrive at 8:17 AM. This clearly shows how the traditional CBTC model stalls under dynamic conditions and highlights the need for an adaptive AI-based headway optimization system.

To overcome the given problem here we used some of the machine learning models to predict the dwell time at each station during the time of the day so that accordingly we could speed up the cascading trains in such a manner that  it doesn’t lead braking of the signalling headway that is issued by signalling department but the running headway can be reduced we used open cv models to process the real time data at station and then getting dwell time at different interval of time .after getting the data I the training of those points were done leading to the prediction of dwell time and then accordingly the train operation was done on real time so that the headway may be optimized .the detailed explanation is written in the report below of how the technologies were used and what were the variables that were taken into consideration making this project make eligible for real life application .

Contents

| Certificate |  |
| --- | --- |
| Acknowledgement |  |
| Abstract |  |
| Contents |  |
| List of figures |  |
| List of tables |  |
| Nomenclature |  |
| Weekly activity |  |

Introduction

This project is undertaken in consultation with Mr. Rashmi Ranjan Patnaik under the General Consultant office, which is responsible for establishing Metro Line 4 and 4A, stretching over 35 km. The station locations are covered from Gaimukh to Bhakti Park, but this project focuses on a specific section between Gaimukh and Cadbury Junction, which spans nearly 10 km.

With increasing urbanization and population density in cities like Mumbai, metro systems face rising demand. Efficient train operations, especially during peak hours, are vital for reducing congestion, improving commuter experience, and maintaining schedule adherence. Delays not only inconvenience passengers but also strain the system’s capacity. Hence, intelligent headway management becomes a critical focus area especially for cities like Mumbai .

The Mumbai Metro Line 4 development project is being carried out by multiple companies in partnership with each other under the leadership of MMRDA (Mumbai Metropolitan Region Development Authority). Companies like DB Engineering, Hill International, and Louis Berger are cooperating among themselves for the development of this project.

Metro Line 4 aims to address the major issue of connectivity from the Thane side to Wadala, as currently, there is no efficient transport system operating in this corridor. Many daily commuters reside near Thane and beyond, so connecting to Bhakti Park will benefit these commuters and help share the load on the Mumbai suburban local trains, which currently serve as the primary mode of connectivity between various parts of Mumbai.

- This project focuses on:

- Prediction of dwell time at stations and identification of peak hours using real-world data.

- Incorporation of civil speed limits and other operational constraints into the simulation.

- Adjustment of follow-up speeds to enable headway optimization between successive trains.

- Monitoring and managing acceleration profiles to ensure they stay within acceptable limits for metro systems.

- Visualization of outcomes using a control chart to evaluate the impact of headway optimization on service flow and delay patterns.

- To identify the causes and effects of cascading delay

- To analyse the limitation of existing fixed headway CBTC system

Technological Aspect of the Project

#### Computer Vision

A key factor in this project was the integration of computer vision with transport operations to accurately predict the dwell time at each station. If dwell time increases at a station, it can lead to cascading delays throughout the line. However, if we can trace the predicted dwell time at each time of day, we can adjust the train's speed accordingly to optimize headway and reduce delays.

To achieve this, the YOLO model (You Only Look Once) was used for person detection in video frames. YOLO is a real-time object detection model that proved to be more effective compared to other models I tried—such as some internal OpenCV models. While the OpenCV models predicted reasonably, they didn’t match the real-world data as accurately as YOLO. The YOLO model was used to extract crowd inflow at different times during the day, which provided useful data for analysing passenger load.

#### Machine Learning

Machine learning was applied to determine how dwell time changes with crowd inflow. After extracting inflow data from YOLO, I tested different ML models to figure out which one best matched the real data captured from the OpenCV-based crowd detection. This model helped predict dwell time dynamically at each station throughout the day. The final predicted dwell times were then fed into the simulation to optimize headway and maintain smooth train operations.

SIMULATION USING C++

Kinematics equations were used to figure out the movement of the train and the civil speed limit data so that the speed restriction is taken into place and all this is done using the data available like the breaking distance and the acceleration profile that is between the parameters that was provided by the mentor so that we could visualize our result and that can be used for the simulation of the train

This project therefore combines real world operational constraints with advanced technologies like computer vision and machine learning to create a realistic and adaptive metro simulation model .By focusing on dwell time predication and headway optimisation and delay control it offers a practical framework for enhancing the efficiency of urban rail system the following section of this report detail the methodology implementation and result derived from the work that was carried out

Methodology

In this section I have described how the technologies were used and what were their outputs

COMPUTER VISION:

The model used in this project is YOLOv5 (You Only Look Once, version 5) for real-time detection of crowd inflow at metro stations during different intervals of the day. Due to the current construction phase of Metro Line 4, actual crowd footage from this corridor was not available. Therefore, video surveillance feed from the operational Metro Line 1 (Ghatkopar to Versova) was used as a proxy dataset to simulate and prototype the computer vision-based crowd detection framework.

The YOLOv5 model was selected after comparative testing with other object detection models (basic OpenCV-based detectors). YOLOv5 outperformed others in terms of detection accuracy, real-time performance, and frame-level consistency, making it suitable for high-frequency detection needed in transport applications.

A screenshot of the detection output is attached below, demonstrating how the YOLOv5 model detects and counts individuals in each frame. This count is then logged with a timestamp and later used as an input feature for machine learning-based dwell time prediction.

YOLO v -5 code used to process the video is attached here

Python code snippet used for yolo v-5 model import torch import cv2 import csv from datetime import datetime # Load YOLOv5s model (small, fast, accurate enough for real-time person detection) model = torch.hub.load('ultralytics/yolov5', 'yolov5s', force_reload=False) model.classes = [0] # Only detect persons (class 0 in COCO dataset) # Initialize video capture (replace with 0 for webcam or use CCTV/IP video file) video_path = 'station_cctv_sample.mp4' # Change this to your video path cap = cv2.VideoCapture(video_path) # Open CSV file for writing person count log csv_file = open('inflow_data.csv', mode='w', newline='') csv_writer = csv.writer(csv_file) csv_writer.writerow(['Timestamp', 'Person_Count']) # Process each frame while cap.isOpened(): ret, frame = cap.read() if not ret: break # Run inference using YOLOv5 results = model(frame) # Extract predictions as pandas dataframe detections = results.pandas().xyxy[0] # Filter for persons person_count = len(detections) # Write timestamp and count to CSV timestamp = datetime.now().strftime("%H:%M:%S") csv_writer.writerow([timestamp, person_count]) # Display results on video cv2.putText(frame, f'Person Count: {person_count}', (20, 40), cv2.FONT_HERSHEY_SIMPLEX, 1, (0, 255, 0), 2) # Show the frame with count overlay cv2.imshow('YOLOv5 Person Detection', frame) # Break on 'q' key press if cv2.waitKey(1) & 0xFF == ord('q'): break # Release everything cap.release() cv2.destroyAllWindows() csv_file.close()

The detailed